# MEMORANDUM

| Memo title | Parametric study of ATR |
| --- | --- |
| Last updated | 20/11/2018 |
| Work package: | WP1, WP4, WP5 |
| Model status | Prelimnary, for Elegancy internal use |
| Author | Mijndert van der Spek |

# Version Control

| Version | Changes | Owner(s) | Date | People informed about the change |
| --- | --- | --- | --- | --- |
| V0 | Initial version | MvdS | 20/11/2018 |  |

# Content

This memo accompanies the ATR model developed by SPL at ETH Zürich. It provides version control, aim of the modelling study, and high-level research methods.

Aim: to develop and ATR process that combines i) high H2 production for a given NG input, ii) low overall CO2 emission and iii) where possible, excess electricity production through use of excess HP steam.

Method:

- Single variable parametric analysis (local sensitivity analysis)

- Selection of ranges for multivariable parametric analysis

- Multivariable parametric analysis (global sensitivity analysis)

- Selection of optimal process design parameters

- Pinch analysis & heat exchanger network design

Constant parameters: gas path pressure levels; prereformer temperature; HTS inlet temperature; steam design pressure/temperature.

Varied parameters: reformer steam input; reformer inlet temperature; reformer oxygen input; HTS steam input ratio; LTS inlet temperature.

Monitored parameters: reformer O/C ratio; reformer S/C ratio; reformer outlet T; CH4 conversion; CO conversion in WGS; excess steam production; net power production/consumption; total H2 production; Total CO2 emission; CO2 recovery.

Initial guess (based on literature) and ranges used for the multivariable parametric analysis. These ranges are based on the initial single variable parametric analysis:

| Variable | Initial guess | Lower limit multivariable analysis | Upper limit multivariable analysis |
| --- | --- | --- | --- |
| Steam flowrate [kmol/hr] | 900 | 800 | 950 |
| Steam-carbon ratio |  |  |  |
| Oxygen flowrate [kmol/hr] | 1100 | 800 | 1200 |
| Oxygen-carbon ratio |  |  |  |
| Reformer inlet temperature [°C] | 750 | 650 | 750 |
| LTS reactor inlet temperature [°C] | 190 | 180 | 240 |
| WGS steam flowrate [kmol/hr] | 2500 | 2000 | 3000 |

Example result (pareto comparing hydrogen production with excess electricity production):

Resulting selected design parameters:

Monitored indicator results for selected design parameters:

| Reformer S/C ratio | 0.77 mol/mol |
| --- | --- |
| Reformer O/C ratio | 0.61 mol/mol |
| Reformer outlet T | 1092°C |
| Methane conversion | 98.57% |
| CO conversion in WGS | 98.65% |
| Excess steam to turbine | 1935 kmol/hr |
| Electricity production | 1998 kW |
| H2 production | 4243 kmol/hr |
| CO2 recovery | 88.6% |